# Schottky Barrier Diodes

Block B: Discrete Semiconductors — Family 2

22 Attributes | DigiKey Family ID: TBD

Hard Logic Filters for Replacement Part Validation

| Attribute | Logic Type | Matching Rule | Engineering Reasoning |
| --- | --- | --- | --- |
| Schottky Technology | Identity | Exact Match | Confirms the diode is a Schottky barrier type, not a standard PN junction. Schottky diodes use a metal-semiconductor junction instead of a PN junction. This gives them fundamentally different characteristics: near-zero reverse recovery time (majority carrier device — no minority carrier storage), lower forward voltage (0.2–0.5V vs 0.7–1.1V for silicon), but higher reverse leakage current. A standard silicon rectifier cannot replace a Schottky where low Vf is required (the 0.4–0.7V difference matters enormously in low-voltage circuits). Conversely, a Schottky cannot replace a standard rectifier in high-voltage applications where leakage current matters — Schottky leakage at 150°C can be orders of magnitude higher. |
| Max Repetitive Peak Reverse Voltage (Vrrm) | Threshold | Replacement ≥ Original | The maximum repetitive reverse voltage the diode can block. Silicon Schottky diodes are typically limited to ≤200V (practical limit ≈100V for good Vf) because the metal-semiconductor barrier breaks down at lower voltages than a PN junction. Above 100V, the Vf advantage over standard silicon shrinks and leakage increases dramatically. SiC (Silicon Carbide) Schottky diodes extend to 600V, 650V, 1200V, 1700V while maintaining the zero-recovery-time advantage. Common silicon Schottky values: 20V, 30V, 40V, 45V, 60V, 100V, 150V, 200V. Higher is always safe, but at higher voltages, verify that the Vf benefit still justifies using Schottky over a standard fast-recovery diode. |
| Average Rectified Forward Current (Io) | Threshold | Replacement ≥ Original | The maximum average forward current at a specified reference temperature. Same principle as rectifier diodes — compare at the same reference temperature (Tc or Ta). Common Schottky current ratings: 0.5A, 1A, 2A, 3A, 5A, 10A, 15A, 20A, 30A, 40A, 60A. Schottky diodes at the low end (≤1A) often come in very small packages (SOD-323, SOD-123) where the package thermal limit determines the current rating, not the die. Higher current rating is always acceptable. |
| Forward Voltage Drop (Vf) | Threshold | Replacement ≤ Original | THE dominant specification for Schottky diodes — this is why they exist. Typical Schottky Vf: 0.2–0.5V at rated current (vs. 0.7–1.1V for standard silicon). Vf is specified at a given forward current and temperature — always compare at the same conditions. Critical trade-off: lower Vf = higher reverse leakage. Schottky diodes optimized for the lowest possible Vf (e.g., 0.2V) have the highest leakage current. This is a fundamental property of the barrier height — a lower barrier makes it easier for electrons to pass in both directions. In low-voltage applications (3.3V, 5V), every 50mV of Vf difference is significant. For Schottky, Vf is almost always the primary selection criterion. |
| Reverse Leakage Current (Ir) | Threshold | Replacement ≤ Original | This is the Achilles' heel of Schottky diodes and a critical matching parameter. Schottky leakage is 10–1000× higher than standard silicon rectifiers at the same voltage and temperature. Typical values: 10µA–1mA at 25°C, rising to 1–100mA at 125°C. Leakage doubles approximately every 10°C. At high temperatures, Schottky leakage current can become a significant power dissipation source: 50mA × 30V = 1.5W of leakage power, which further heats the device — creating a thermal runaway risk similar to MOVs. This is why Schottky diodes are rarely used above 100–150V in silicon: the leakage becomes unmanageable. In battery-powered circuits, leakage directly drains the battery. A replacement with higher leakage may be electrically functional but thermally or energy-budget unacceptable. |
| Max Surge Forward Current (Ifsm) | Threshold | Replacement ≥ Original | The maximum non-repetitive peak forward current for a single half-sine pulse (typically 8.3ms or 10ms). Same purpose as rectifier diodes — handles capacitor inrush at power-on. Schottky diodes generally have comparable Ifsm to silicon rectifiers of the same die size. A replacement with lower Ifsm may survive steady-state but fail on startup. |
| Junction Capacitance (Cj) | Threshold | Replacement ≤ Original | The capacitance of the metal-semiconductor junction when reverse-biased. Schottky diodes are frequently used in high-frequency switching circuits specifically because they have no reverse recovery charge — but junction capacitance still limits switching speed. The capacitance must be charged and discharged every switching cycle, contributing to switching losses. At 1MHz+, Cj becomes a primary differentiator between Schottky diodes. Typical values: 10–200pF for power Schottky, 0.5–5pF for small-signal/RF Schottky. Lower Cj = faster switching and lower capacitive losses. Elevated from Application Review (rectifier diodes) to Threshold for Schottky because Schottky diodes are disproportionately used in high-frequency applications where Cj matters. |
| Configuration | Identity | Exact Match | Same principle as rectifier diodes. Schottky configurations include: Single (one diode per package), Dual Common Cathode (very common — two Schottky diodes sharing a cathode for full-wave center-tap rectifiers in switching supplies), Dual Common Anode (for OR-ing supplies), Dual Series (half-bridge). Dual common cathode is the single most popular Schottky configuration because of its use in switching power supply output rectification. The replacement MUST match the configuration exactly — internal wiring determines pin-to-function mapping. |
| Package / Form Factor | Identity | Exact Match to Original | Must match the PCB footprint. Common Schottky packages include all the standard rectifier packages plus some Schottky-specific ones: SOD-323 (very small, ≤1A), SOD-123 (small, ≤1A), SMA/DO-214AC, SMB/DO-214AA, SMC/DO-214AB, DPAK/D²PAK (high power), PowerDI 5, PowerDI 3333. Through-hole: DO-41, DO-15, DO-201, TO-220 (high power), TO-247. Some manufacturers have proprietary power packages optimized for low-Vf Schottky (e.g., Vishay's TMBS series in SMP, Diodes Inc's PowerDI). Verify the exact footprint including any exposed pad dimensions. |
| Pin Configuration / Polarity Marking | Identity | Exact Match | Identical concern to rectifier diodes. For single diodes in 2-pin packages, straightforward (cathode band). For dual packages, the pin-to-function mapping varies between manufacturers. A dual common-cathode Schottky in a SOT-23 package has three pins — and different manufacturers may assign them differently. Some SOT-23 Schottky duals have anode1-common cathode-anode2, others have anode1-anode2-common cathode. Always verify from the datasheet. Getting this wrong on a Schottky is especially dangerous because Schottky diodes have much lower reverse voltage tolerance than silicon — a reverse-connected Schottky will fail immediately. |
| Thermal Resistance, Junction-to-Case (Rθjc) | Threshold | Replacement ≤ Original | How effectively the diode transfers heat from junction to case. Lower is better. Thermal management is more critical for Schottky than standard rectifiers because Schottky leakage current increases exponentially with temperature. Poor thermal design creates a positive feedback loop: leakage → heating → more leakage → more heating. The thermal resistance must be low enough that the junction temperature stays in a range where leakage is manageable. For heatsink-mounted packages (TO-220, TO-247, DPAK), Rθjc is the dominant metric. |
| Thermal Resistance, Junction-to-Ambient (Rθja) | Threshold | Replacement ≤ Original | Relevant for SMD packages relying on PCB copper for heat dissipation. Same principle as rectifier diodes, but the thermal runaway risk from Schottky leakage makes this more important. Ensure the replacement's Rθja is adequate to keep the junction temperature in a safe range, accounting for both conduction loss AND leakage power dissipation at the operating temperature. |
| Max Junction Temperature (Tj_max) | Threshold | Replacement ≥ Original | Standard silicon Schottky: typically 125°C or 150°C. SiC Schottky: 175°C. Note that many silicon Schottky diodes have a lower Tj_max than standard silicon rectifiers (125°C vs 150–175°C) specifically because of the leakage/thermal runaway concern — the manufacturer limits Tj to keep leakage manageable. A replacement with lower Tj_max has less thermal headroom. Higher is always acceptable. |
| Operating Temperature Range | Threshold | Replacement range ⊇ Original range | The replacement range must fully contain the original. Typical: -55°C to +125°C or -65°C to +150°C. The upper end is where Schottky diodes get into trouble — leakage at 125°C can be 100× the 25°C value. For automotive and industrial applications, verify that the leakage current at the upper ambient temperature is still acceptable. |
| Power Dissipation (Pd) | Threshold | Replacement ≥ Original | Maximum power at a given reference temperature. For Schottky diodes, total Pd includes: conduction loss (Vf × If), leakage loss (Ir × Vr, significant at high temperatures), and capacitive switching loss (½ × Cj × V² × f). At high temperatures, the leakage loss can approach or exceed the conduction loss — this is unique to Schottky. A replacement with lower Pd may not handle the combined thermal load. |
| Technology (Trench vs. Planar) | Application Review | Verify trade-offs acceptable | Modern Schottky diodes use two main silicon technologies: Trench Schottky and Planar Schottky. Trench technology offers lower Vf and better Ir vs. Vf trade-off — for the same leakage current, a trench Schottky has lower Vf, or for the same Vf, lower leakage. Planar is the older technology. If the original is a trench Schottky (often marketed as TMBS, Trench MOS Barrier Schottky, or similar), replacing with a planar Schottky of the same Vf will likely have higher leakage, and vice versa. Not all datasheets specify the technology — flag for review if the Vf/Ir ratio differs significantly between original and replacement. |
| Semiconductor Material (Si vs. SiC) | Identity (Flag) | If Original = SiC, Replacement = SiC | Silicon Carbide (SiC) Schottky diodes are a distinct technology class from silicon Schottky. SiC advantages: much higher voltage ratings (600–1700V), virtually zero reverse recovery (even better than Si Schottky), temperature-stable leakage current, higher Tj_max (175°C). SiC disadvantages: higher cost (3–10× silicon), higher Vf at low currents (1.2–1.7V — worse than silicon Schottky below ~5A). A silicon Schottky CANNOT replace a SiC Schottky at high voltage — silicon Schottky simply doesn't exist above 200V. A SiC Schottky CAN replace a silicon Schottky (it's an upgrade) but the higher Vf at low currents may be unacceptable in low-voltage applications. Flag any Si↔SiC swap for review. |
| Vf Temperature Coefficient | Application Review | Verify for current-sharing applications | Schottky diodes have a positive temperature coefficient of Vf at high currents (Vf increases with temperature) but a negative temperature coefficient at low currents (Vf decreases with temperature). The crossover point depends on the die design. This matters for two scenarios: (1) Parallel operation — if two Schottky diodes are paralleled, the positive tempco at high current provides natural current sharing (the hotter diode has higher Vf, so current shifts to the cooler one). If operating in the negative tempco region, thermal runaway can occur. (2) Wide temperature range operation — Vf varies significantly with temperature, affecting efficiency calculations. Flag for review if parallel operation or wide temp range is involved. |
| Mounting Style | Identity | Exact Match | Surface mount vs. through-hole. Cannot interchange without PCB redesign. Same considerations as rectifier diodes — power pad packages require matching PCB thermal pads. Schottky diodes in SOD-323 and SOD-123 packages are extremely common for low-current applications; verify the exact sub-variant. |
| Height (Seated Max) | Fit | Replacement ≤ Original | Verify mechanical clearance. Same principle as rectifier diodes — check enclosure, stacked PCB, and adjacent component clearances. |
| AEC-Q101 Qualification | Identity (Flag) | If Original = Yes, Replacement = Yes | Automotive qualification for discrete semiconductors. AEC-Q101 (not AEC-Q200, which is for passives). Schottky diodes in automotive are used extensively for reverse polarity protection, voltage clamping, power supply rectification, and flyback/freewheeling. A non-qualified part cannot replace a qualified one. |
| Packaging (Tape & Reel / Tube / Bulk) | Operational | Production Match | SMD: Tape & Reel for automated assembly. Through-hole: Ammo Pack or Tube for auto-insertion, Bulk for hand assembly. Mismatch doesn't affect performance but can halt production. |

## Schottky Diode-Specific Notes

Why Schottky Has No Recovery Time Attributes: Schottky diodes are majority-carrier devices — current flows by electrons crossing a metal-semiconductor barrier, not by minority carrier injection across a PN junction. When the voltage reverses, there are no stored minority carriers to "recover." The reverse current during switching is determined only by junction capacitance (Cj), not by stored charge. This is why trr, Qrr, and recovery behavior (soft/snappy) — all present in the rectifier diode table — are absent from this table. The switching speed of a Schottky diode is limited only by its junction capacitance, which makes them inherently suited for high-frequency circuits. This is the fundamental reason Schottky diodes exist as a product category.

The Vf vs. Ir Trade-Off — The Schottky Triangle: Every Schottky diode designer faces three competing goals: low Vf, low Ir, and high Vrrm. You can optimize for any two but not all three. A lower Schottky barrier height reduces Vf but increases leakage (Ir) because the lower barrier is easier for electrons to cross in both directions. A higher barrier reduces leakage but increases Vf. Higher voltage ratings require thicker, more resistive epitaxial layers that increase Vf. This is why low-voltage Schottky diodes (20–40V) have the best Vf/Ir ratios, and why high-voltage Schottky (≥100V) barely outperforms standard silicon on Vf while having much worse leakage. The matching engine should understand that a replacement with lower Vf almost certainly has higher Ir, and the assessment should flag this trade-off.

Thermal Runaway Risk Is Real: This is the most dangerous failure mode unique to Schottky diodes and something the assessment should always mention when leakage is a concern. The sequence: (1) Leakage current at rated reverse voltage generates heat. (2) Junction temperature rises. (3) Leakage approximately doubles every 10°C. (4) More heat → more leakage → more heat. If the thermal resistance is too high or the ambient temperature is too warm, this positive feedback loop leads to junction temperature exceeding Tj_max and catastrophic failure. The risk is highest at high reverse voltage and high ambient temperature. The matching engine should flag any replacement where the leakage at max operating temperature (calculated using the Ir temperature coefficient) exceeds the original's, because this directly affects thermal stability margin.

SiC Schottky — A Different Product Category: Silicon Carbide Schottky diodes solve most of the limitations of silicon Schottky — they handle 600V+ with low leakage and temperature-stable characteristics — but at 3–10× the cost and with higher Vf at low currents (1.2–1.7V). SiC Schottky diodes are primarily used in 400V+ PFC (Power Factor Correction) stages and 600V+ half-bridge rectification where they replace silicon fast-recovery diodes, not silicon Schottky. The Part Resolution Service should classify SiC Schottky as a distinct sub-family because the matching priorities are different: Vf is no longer the lowest (it's comparable to ultrafast silicon), but zero recovery and temperature stability are the advantages.

Comparison to Rectifier Diode Table — What Changed: Removed: Recovery Category, trr, Qrr, Recovery Behavior (soft/snappy) — Schottky has no reverse recovery. Added: Schottky Technology (Identity gate), Semiconductor Material Si/SiC (Identity Flag), Technology Trench/Planar (Application Review), Vf Temperature Coefficient (Application Review). Changed: Junction Capacitance elevated from Application Review to Threshold (Schottky are used in high-frequency circuits where Cj matters). Vf reasoning emphasized as THE dominant spec. Ir reasoning expanded significantly due to thermal runaway risk. Vdc attribute removed — Schottky diodes typically specify only Vrrm, and Vdc is assumed equal.

## Logic Type Summary

Identity (6): Schottky Technology, Configuration, Package, Pin Configuration, Mounting Style, AEC-Q101 (Flag), SiC vs Si (Flag)

Threshold (10): Vrrm (≥), Io (≥), Vf (≤), Ir (≤), Ifsm (≥), Cj (≤), Rθjc (≤), Rθja (≤), Tj_max (≥), Pd (≥)

Threshold — Range (1): Operating Temperature Range (⊇)

Fit (1): Height

Application Review (2): Technology (Trench/Planar), Vf Temperature Coefficient

Operational (1): Packaging (Tape & Reel / Tube / Bulk)