# Varistors / Metal Oxide Varistors (MOVs)

DigiKey Family ID: 65

Hard Logic Filters for Replacement Part Validation

| Attribute | Logic Type | Matching Rule | Engineering Reasoning |
| --- | --- | --- | --- |
| Varistor Voltage (V₁ₘₐ) | Identity | Exact Match | This is the primary specification. The voltage at which the varistor begins to conduct (typically specified at 1mA DC). A 275V varistor must be replaced by a 275V varistor. This value determines the clamping threshold — too low trips during normal operation; too high fails to protect. Common values track mains voltages: 180V, 275V, 320V, 460V for AC; various DC ratings for automotive/industrial. |
| Clamping Voltage (Vc) | Threshold | Replacement ≤ Original | The voltage across the varistor when conducting the specified peak current. Lower clamping voltage provides better protection for downstream components. Specified at a given current pulse (e.g., Vc = 710V at 100A (8/20µs)). The replacement must clamp at the same or lower voltage at the same test current. A higher clamping voltage allows more energy through to the protected circuit. |
| Maximum Continuous Voltage (AC/DC) | Threshold | Replacement ≥ Original | The maximum continuous operating voltage the varistor can withstand without significant leakage or degradation. Specified as both AC RMS and DC. The replacement must handle at least the same continuous voltage. A lower-rated varistor will draw excessive leakage current under normal operating conditions, causing self-heating and premature failure. |
| Energy Rating (Joules) | Threshold | Replacement ≥ Original | The maximum single-pulse energy the varistor can absorb without damage, typically specified for a 10/1000µs waveform or 2ms pulse. Higher energy rating means more robust surge protection. A replacement with lower energy rating may fail during a surge event that the original would have survived. Typical range: 0.1J (small SMD) to 1000J+ (large disc). |
| Peak Surge Current (8/20µs) | Threshold | Replacement ≥ Original | The maximum peak current the varistor can handle for a single 8/20µs industry-standard lightning surge waveform. This is the standard surge test waveform (8µs rise, 20µs to half-value). Higher is better. Typical range: 50A (small SMD) to 70,000A (large disc). If the application requires compliance with a specific surge standard (IEC 61000-4-5), the replacement must meet the required test level. |
| Package / Form Factor | Identity | Exact Match to Original | Varistors come in: radial disc (through-hole) — the most common for power-line protection; SMD (chip) — for board-level ESD/transient protection; block/strap — for high-energy industrial applications. The replacement must match the original mounting style and footprint. |
| Disc Diameter (Radial) | Fit | Replacement ≤ Original | For radial disc MOVs, the disc diameter directly correlates with energy and surge current ratings. Common sizes: 5mm, 7mm, 10mm, 14mm, 20mm, 25mm, 32mm, 40mm. A larger disc generally has better surge handling, but must physically fit the board space and lead spacing. A smaller disc with equivalent ratings is acceptable if it fits. |
| Lead Spacing / Pitch | Identity | Exact Match | For through-hole radial disc varistors, lead spacing must match the PCB hole pattern. Standard pitches: 5mm, 7.5mm, 10mm, 12.5mm. Lead spacing is tied to disc diameter. A different disc size usually means a different pitch. |
| Response Time | Threshold | Replacement ≤ Original | The time for the varistor to begin clamping after a transient event. MOVs are inherently fast (<25ns for the ZnO element, <500ns including leads). SMD types are faster than through-hole due to shorter leads. Response time is rarely a differentiator between MOVs of the same form factor, but verify if replacing a fast SMD type with a through-hole type. |
| Leakage Current | Threshold | Replacement ≤ Original | The current drawn by the varistor at its maximum continuous operating voltage. Lower is better. Excessive leakage causes self-heating, which increases leakage further — a positive feedback loop that leads to thermal runaway. Typical values: <20µA for standard MOVs, <1µA for low-leakage types. Particularly important in battery-powered or energy-harvesting applications. |
| Operating Temp Range | Threshold | Replacement range ⊇ Original range | The replacement must cover at least the full operating range. Standard MOVs: −40°C to +85°C. Varistor performance degrades at high temperatures — leakage current increases and clamping voltage shifts. A narrower range cannot substitute for a wider one. |
| Number of Surge Pulses (Lifetime) | Threshold | Replacement ≥ Original | MOVs degrade with each surge event — the clamping voltage drifts lower and leakage current increases. Rated surge life is the number of pulses at a specified level before the varistor voltage shifts by a defined percentage. Applications with frequent transients (industrial environments, motor loads) need higher pulse lifetime. More pulses is better. |
| Safety Rating (UL, IEC) | Identity (Flag) | If Original = Certified, Replacement = Certified | Varistors in mains-connected surge protection circuits typically require UL 1449 (SPD) or IEC 61643 certification. Thermal disconnect (built-in thermal fuse) may be required for UL listing. A non-certified part cannot replace a certified one in mains-connected applications. |
| Thermal Disconnect / Fuse | Identity (Flag) | If Original = Yes, Replacement = Yes | Some MOVs have a built-in thermal disconnect (thermal fuse) that opens the circuit if the varistor overheats due to sustained overvoltage or end-of-life degradation. This prevents a catastrophic failure (fire). Required by UL 1449 for certain applications. If the original has a thermal disconnect, the replacement must also have one. |
| AEC-Q200 Qualification | Identity (Flag) | If Original = Yes, Replacement = Yes | Required for automotive applications. Varistors in automotive protect against load dump transients, inductive spikes, and ESD. AEC-Q200 qualified parts undergo rigorous testing. A non-qualified part cannot replace a qualified one. |
| Packaging | Operational | Production Match | For SMD varistors, Tape & Reel is required for automated placement. For through-hole disc types, Ammo Pack or Bulk depending on the assembly process. |

## Varistor-Specific Notes

MOVs Degrade With Every Surge: Unlike most passive components that either work or fail, MOVs gradually degrade with each transient event. Each surge causes micro-structural damage in the zinc oxide grains, slightly lowering the varistor voltage and increasing leakage current. After enough surges, the varistor's leakage becomes significant enough to cause self-heating, which accelerates degradation in a positive feedback loop. End-of-life is often defined as varistor voltage shifting by more than 10%. This is why surge pulse lifetime matters — in applications with frequent transients (industrial motors, inductive loads), the MOV may reach end-of-life in months, not years.

Thermal Runaway Risk: If an MOV is subjected to sustained overvoltage (not a brief transient, but a persistent voltage above its maximum continuous rating), it will draw increasing leakage current, causing self-heating, which increases leakage further. This thermal runaway can result in fire. This is why UL 1449 requires thermal disconnects in SPD (Surge Protective Device) applications — the thermal fuse opens the circuit before the MOV reaches ignition temperature. When substituting, verify that the thermal disconnect mechanism is present if the original had one. A bare MOV without a thermal disconnect cannot replace one with a disconnect in a mains-connected application.

Clamping Voltage vs. Varistor Voltage: These are different specs that are often confused. Varistor voltage (V₁ₘₐ) is the voltage at which the device begins to conduct (at 1mA, a very small current). Clamping voltage (Vc) is the voltage across the device when it's conducting a large surge current (e.g., 100A). The clamping voltage is always higher than the varistor voltage — it's the actual voltage the protected circuit sees during a surge. For protection design, the clamping voltage is what matters. Two MOVs with the same varistor voltage can have different clamping voltages depending on their dynamic resistance.