# Aluminum Electrolytic Capacitors

DigiKey Family ID: 58

Hard Logic Filters for Replacement Part Validation

Things that are specific to this component family:

- Ripple Current Rating — arguably the single most important reliability parameter for electrolytics. Exceeding it causes internal heating and accelerated electrolyte dry-out, leading to premature failure.

- Lifetime / Endurance — unique to electrolytics since the wet electrolyte evaporates over time. Rated in hours at max temp, with the Arrhenius doubling rule explained in the notes.

- Leakage Current — inherent to aluminum oxide dielectrics and critical for battery-powered or high-impedance circuits.

- Dissipation Factor (tan δ) — more relevant here than in ceramics due to the lossy electrolyte.

- Polarization — treated as exact match since substituting a polarized cap for a bipolar one (or vice versa) can cause catastrophic failure.

- Lead Spacing — a constraint that doesn't exist for SMD ceramics but is critical for through-hole electrolytics, and it's coupled to diameter.

- Diameter + Height — both as separate fit constraints, since electrolytics are physically much larger and more variable in size than MLCCs.

- Capacitor Type/Series — captures the sub-category hierarchy (standard → low-ESR → long-life, etc.)

| Attribute | Logic Type | Matching Rule | Engineering Reasoning |
| --- | --- | --- | --- |
| Capacitance | Identity | Exact Match | A 100µF capacitor must be replaced by a 100µF capacitor. Normalize values before comparison (e.g., 0.1mF = 100µF). Electrolytic values follow the E-series but with wider steps than ceramics. |
| Voltage Rating | Threshold | Replacement ≥ Original | The replacement must handle at least the same voltage. A 50V cap can replace a 25V one. Note: higher voltage-rated aluminum electrolytics are physically larger, so also verify the diameter and height fit constraints. |
| Polarization | Identity | Exact Match | Most aluminum electrolytics are polarized. Bipolar (non-polar) variants exist for AC-coupled or signal-path applications. A polarized cap cannot replace a bipolar one (reverse voltage will destroy it). A bipolar cap can technically replace a polarized one, but is often larger and more expensive—treat as exact match to avoid unnecessary substitution. |
| Mounting Type | Identity | Exact Match | Aluminum electrolytics come in through-hole (radial/axial) and SMD (V-chip) packages. These are not interchangeable. A radial leaded cap cannot replace an SMD cap without board rework. Match the mounting type exactly. |
| Diameter (Can Size) | Fit | Replacement ≤ Original | The can diameter must fit within the PCB footprint and not encroach on adjacent components. For through-hole, the lead spacing is tied to diameter. For SMD, the pad pattern is diameter-specific. A smaller or equal diameter is safe; a larger one may not fit. Common sizes: 5mm, 6.3mm, 8mm, 10mm, 12.5mm, 16mm. |
| Height (Seated Max) | Fit | Replacement ≤ Original | Critical for enclosure clearance. Aluminum electrolytics are among the tallest components on a board. Higher capacitance and voltage ratings generally mean taller cans. Always verify height when substituting with a higher voltage-rated part. |
| Lead Spacing (Through-Hole) | Identity | Exact Match | For radial leaded parts, the lead spacing (pitch) must match the PCB hole pattern. Common pitches: 2mm, 2.5mm, 3.5mm, 5mm, 7.5mm. Lead spacing is directly tied to can diameter—a different diameter often means a different pitch, creating a double constraint. |
| Tolerance | Threshold | Replacement ≤ Original | A tighter tolerance is always acceptable. Aluminum electrolytics typically have wide tolerances: ±20% (M) is standard. Some precision or low-cost parts may be rated at +50%/−20% (asymmetric). If the original has an asymmetric tolerance, ensure the replacement covers at least the same range. |
| ESR (Equivalent Series Resistance) | Threshold | Replacement ≤ Original | ESR is one of the most critical parameters for aluminum electrolytics, especially in power supply output filtering. High ESR causes voltage ripple and heat generation. Low-ESR series (e.g., Panasonic FR, Nichicon UHE) are not interchangeable with standard series. Always compare ESR at the same frequency and temperature (typically 100kHz, 20°C). |
| Ripple Current Rating | Threshold | Replacement ≥ Original | The maximum RMS ripple current the cap can handle without exceeding its thermal limits. This is the #1 reliability driver for electrolytics. Exceeding the ripple current rating causes internal heating, accelerated electrolyte evaporation, and premature failure. A 2A rated cap can replace a 1.5A one. Always compare at the same frequency and temperature. |
| Impedance (at freq.) | Threshold | Replacement ≤ Original | Impedance combines ESR and reactance at a given frequency. Lower impedance means better high-frequency filtering. Particularly important at 100kHz for switching power supply applications. Some datasheets specify impedance instead of (or in addition to) ESR—use whichever is available for comparison. |
| Dissipation Factor (tan δ) | Threshold | Replacement ≤ Original | A measure of energy loss in the capacitor, typically specified at 120Hz and 20°C. Lower is better. Standard aluminum electrolytics range from 0.05 to 0.30 depending on capacitance and voltage. A higher dissipation factor means more heat generation under AC ripple. |
| Leakage Current | Threshold | Replacement ≤ Original | Aluminum electrolytics have inherent DC leakage through the oxide dielectric. Lower is better. Typically specified as I = 0.01CV or 3µA (whichever is greater) after 2–5 minutes of applied voltage. Critical in battery-powered, sleep-mode, or high-impedance circuits where leakage drains energy or introduces offset. |
| Lifetime / Endurance | Threshold | Replacement ≥ Original | Rated in hours at maximum temperature (e.g., 2000 hrs at 105°C or 10,000 hrs at 105°C). This is the expected useful life before electrolyte dry-out causes capacitance loss and ESR rise. Longer is better. Lifetime roughly doubles for every 10°C below the rated temperature (Arrhenius rule). |
| Operating Temp (Max) | Threshold | Replacement ≥ Original | Standard grades are 85°C and 105°C; some high-reliability parts are rated to 125°C or 150°C. A 105°C cap can replace an 85°C one (and will also last longer at the same operating temp due to the Arrhenius relationship). Never downgrade the temperature rating. |
| Capacitor Type / Series | Identity (Upgrade Only) | Replacement = or ↑ Original | Within aluminum electrolytics, there are sub-categories: standard, low-ESR, low-leakage, high-ripple, and long-life. A low-ESR cap can replace a standard one (upgrade), but a standard cap cannot replace a low-ESR one if the circuit depends on low ESR for stability or ripple performance. |
| AEC-Q200 Qualification | Identity (Flag) | If Original = Yes, Replacement = Yes | Required for automotive applications. Aluminum electrolytics are often the weakest link in automotive reliability due to electrolyte evaporation under heat and vibration. AEC-Q200 qualified parts undergo extended life, vibration, and humidity testing. A non-qualified part cannot replace a qualified one. |
| Moisture Sensitivity Level (MSL) | Threshold | Replacement ≤ Original | Primarily relevant for SMD aluminum electrolytics. MSL 1 (unlimited floor life) is best. Through-hole parts are generally not MSL-sensitive, but SMD V-chip electrolytics may have MSL ratings that affect assembly handling and baking requirements. |
| Packaging (Tape & Reel, etc.) | Operational | Production Match | For SMD electrolytics, Tape & Reel is required for automated pick-and-place. For through-hole, Ammo Pack (taped & formed leads) is needed for auto-insertion machines. Bulk packaging requires manual or custom handling. Ensure compatibility with your production line. |

## Logic Type Definitions

Identity: The replacement value must exactly match the original. No deviation allowed.

Identity (Upgrade Only): Must match or be a strictly superior variant (e.g., standard → low-ESR is an upgrade, but not the reverse).

Identity (Flag): A boolean attribute. If the original requires it, the replacement must also have it. If the original does not, either value is acceptable.

Threshold: A numeric attribute where the replacement must meet or exceed (or be at/below) a boundary defined by the original part’s spec.

Fit: A physical/dimensional constraint. The replacement must physically fit in the same space.

Operational: A non-electrical parameter relevant to manufacturing or supply chain compatibility.

## Aluminum Electrolytic-Specific Notes

Arrhenius Lifetime Rule: Electrolytic capacitor lifetime approximately doubles for every 10°C reduction in operating temperature below the rated maximum. A cap rated 2,000 hrs at 105°C will last roughly 8,000 hrs at 85°C. This means a 105°C-rated part is always a meaningful upgrade over an 85°C-rated part, even if the endurance hours are the same.

Reforming After Storage: Aluminum electrolytics that sit unused for extended periods (>6–12 months) may require voltage reforming to restore the oxide dielectric layer. Leakage current increases during storage. This is a consideration for replacement parts pulled from long-term inventory—verify shelf life and storage conditions.

Vibration Sensitivity: Aluminum electrolytics are heavy and top-heavy, making them vulnerable to vibration-induced lead fatigue and solder joint cracking. In automotive, industrial, or aerospace applications, verify that the replacement has equivalent or better vibration ratings. Some manufacturers offer reinforced base or adhesive-mount variants for high-vibration environments.

Ripple Current Frequency Multipliers: Ripple current ratings are typically specified at 100–120Hz. At higher switching frequencies (e.g., 100kHz in SMPS), manufacturers publish frequency multiplier coefficients (often 1.4–1.7×). When comparing ripple current ratings between parts, ensure both values are at the same reference frequency, or apply the appropriate multiplier.