# Chip Resistors – Surface Mount

DigiKey Family ID: 52

Hard Logic Filters for Replacement Part Validation

| Attribute | Logic Type | Matching Rule | Engineering Reasoning |
| --- | --- | --- | --- |
| Resistance | Identity | Exact Match | A 10kΩ resistor must be replaced by a 10kΩ resistor. Ensure values are normalized (e.g., 10k = 10000Ω) before comparison. |
| Package / Case | Identity | Exact Match to Original | The replacement must match the original footprint exactly (e.g., if the original is 0805, the replacement must also be 0805). Different packages have different pad geometries and cannot be interchanged without a board redesign. |
| Composition | Identity (Upgrade Only) | Replacement = or ↑ Original | Thick Film is standard. Thin Film is high precision. You can upgrade Thick → Thin, but not downgrade Thin → Thick. If original specifies Thin Film, replacement must also be Thin Film. |
| Tolerance | Threshold | Replacement ≤ Original | A tighter tolerance is always acceptable. A ±1% resistor can replace a ±5% one, but not vice versa. Lower value = better precision. |
| Power Rating (Watts) | Threshold | Replacement ≥ Original | The replacement must handle at least as much power dissipation. A 0.25W resistor can safely replace a 0.125W one (assuming it physically fits). Higher wattage = safer. |
| Voltage Rating (Max) | Threshold | Replacement ≥ Original | Often overlooked but critical for safety. Smaller packages have lower voltage limits (e.g., an 0402 may be rated 50V while an 0805 handles 150V). Exceeding the rating risks arcing or dielectric breakdown. |
| Temperature Coefficient (TCR) | Threshold | ¦Replacement¦ ≤ ¦Original¦ | Expressed as ±ppm/°C. ±25ppm/°C is better (more stable) than ±100ppm/°C. Compare absolute magnitudes: closer to zero is better. Critical in precision analog circuits. |
| Operating Temp (Max) | Threshold | Replacement ≥ Original | If the board operates at high temperatures (e.g., 125°C), the replacement must be rated to survive at least that. Higher max operating temp = more robust. |
| Height (Seated Max) | Fit | Replacement ≤ Original | Critical for tight enclosures and stacked PCB assemblies. A taller part may physically interfere with enclosure walls or adjacent boards. Shorter or equal is always safe. |
| Moisture Sensitivity Level (MSL) | Threshold | Replacement ≤ Original | Affects manufacturing/assembly handling. MSL 1 (unlimited floor life) is best; MSL 6 requires baking before reflow. A lower MSL number = less restrictive handling. Do not replace MSL 1 with MSL 3. |
| AEC-Q200 Qualification | Identity (Flag) | If Original = Yes, Replacement = Yes | Required for automotive and high-reliability applications. A non-qualified part cannot replace a qualified one. If the original does not require AEC-Q200, a qualified replacement is acceptable. |
| Anti-Sulfur | Identity (Flag) | If Original = Yes, Replacement = Yes | Anti-sulfur resistors use sulfur-resistant electrode materials. In harsh environments (industrial, automotive), replacing an anti-sulfur part with a standard one risks open-circuit failure due to silver sulfide corrosion. |
| Packaging (Tape & Reel, etc.) | Operational | Production Match | Not an electrical parameter, but critical for manufacturing. Tape & Reel is required for automated pick-and-place. Cut Tape or Bulk may not be compatible with production lines. |

## Logic Type Definitions

Identity: The replacement value must exactly match the original. No deviation allowed.

Identity (Upgrade Only): Must match or be a strictly superior variant (e.g., Thick Film → Thin Film).

Identity (Flag): A boolean attribute. If the original requires it, the replacement must also have it. If the original does not, either value is acceptable.

Threshold: A numeric attribute where the replacement must meet or exceed (or be at/below) a boundary defined by the original part’s spec.

Fit: A physical/dimensional constraint. The replacement must physically fit in the same space.

Operational: A non-electrical parameter relevant to manufacturing or supply chain compatibility.