# Film Capacitors

DigiKey Family ID: 64

Hard Logic Filters for Replacement Part Validation

What makes this family distinct:

- Separate DC and AC voltage ratings — the AC rating is not derivable from DC and must be matched independently. Critical for motor-run, PFC, and mains filter applications.

- Dielectric hierarchy (PP > PPS > PEN > PET) — four tiers instead of the two-tier hierarchy in MLCCs or tantalums, each with very different loss, density, and thermal characteristics.

- Self-healing and construction type (metallized vs. film/foil) — treated as two separate attributes because they interact but aren't the same thing. Self-healing is a safety/reliability property; construction type determines pulse handling capability.

- dV/dt and peak pulse current — parameters that don't exist in any of the other capacitor tables. These are the dominant specs for snubber and pulse discharge applications.

- Safety ratings (X/Y class) — a regulatory compliance requirement unique to film caps used in mains EMI suppression. This isn't optional — it's legally required and has a strict hierarchy (X1 > X2 > X3, Y1 > Y2 > Y3 > Y4).

- Flammability rating (UL 94 V-0) — another regulatory flag, non-negotiable in mains-connected applications.

| Attribute | Logic Type | Matching Rule | Engineering Reasoning |
| --- | --- | --- | --- |
| Capacitance | Identity | Exact Match | A 100nF film capacitor must be replaced by a 100nF film capacitor. Normalize values before comparison (e.g., 0.1µF = 100nF). Film capacitors are available in standard E-series values, typically from 100pF to hundreds of µF depending on dielectric type. |
| Voltage Rating (DC) | Threshold | Replacement ≥ Original | The replacement must handle at least the same DC voltage. A 630V cap can replace a 400V one. Film capacitors are often used in high-voltage applications (power supplies, motor drives, snubbers) where voltage rating is a primary selection criterion. Note: higher voltage ratings generally mean a larger physical size. |
| Voltage Rating (AC) | Threshold | Replacement ≥ Original | If the original specifies an AC voltage rating (common in motor-run, power factor correction, and EMI filter applications), the replacement must meet or exceed it. AC voltage rating is not simply DC rating ÷ √2—it accounts for continuous dielectric stress, self-heating from AC losses, and corona onset. Always match the explicitly stated AC rating, not a derived value. |
| Dielectric Material | Identity (Upgrade Only) | Replacement = or ↑ Original | Film dielectrics have a performance hierarchy. Polypropylene (PP/CBB) is the highest performance: lowest loss (tan δ), best frequency response, excellent temperature stability. Polyester (PET/Mylar) offers higher capacitance density but with higher losses and worse temperature stability. PEN (Polyethylene Naphthalate) sits between the two. PPS (Polyphenylene Sulfide) offers excellent thermal stability up to 150°C. Upgrade path: PP > PPS > PEN > PET. You can replace PET with PP (upgrade), but never replace PP with PET (downgrade). |
| Tolerance | Threshold | Replacement ≤ Original | A tighter tolerance is always acceptable. A ±5% cap can replace a ±10% one. Film capacitors commonly come in ±1% (F), ±2% (G), ±5% (J), ±10% (K), and ±20% (M). Polypropylene types are available in very tight tolerances (±1%) for resonant circuits and precision timing. |
| Package / Case | Identity | Exact Match to Original | Film capacitors come in many form factors: radial box (MKT, MKP), axial tubular, SMD, and snap-in / screw-terminal (for large power film caps). The replacement must match the original mounting style and footprint exactly. Lead spacing (pitch) is critical for through-hole radial types. |
| Lead Spacing / Pitch | Identity | Exact Match | For through-hole film capacitors (the majority of film caps), lead spacing must match the PCB hole pattern. Common pitches: 5mm, 7.5mm, 10mm, 15mm, 22.5mm, 27.5mm. Lead spacing is tied to body width—a physically larger cap at a different voltage or dielectric may have a different pitch even at the same capacitance. |
| Self-Healing | Identity (Flag) | If Original = Yes, Replacement = Yes | Metallized film capacitors are self-healing: a dielectric breakdown vaporizes the thin metal electrode around the fault, isolating the defect and restoring function (with minor capacitance loss). Film/foil capacitors use thicker metal electrodes and do not self-heal—a breakdown is permanent. If the original is metallized (self-healing), the replacement must also be metallized. Film/foil types have lower ESR and handle higher pulse currents, so replacing a film/foil with metallized may reduce pulse handling capability. |
| Construction (Metallized vs. Film/Foil) | Identity (Upgrade Only) | Replacement = or ↑ Original | Metallized construction: thinner, lighter, self-healing, lower pulse current handling. Film/foil: bulkier, no self-healing, but excellent pulse handling and lower ESR. Metallized + film/foil hybrid: combines some benefits of both. For continuous-duty AC applications, metallized is standard. For high-pulse or snubber applications, film/foil is preferred. Do not replace film/foil with metallized in high-pulse circuits without verifying dV/dt and peak current specs. |
| Dissipation Factor (tan δ) | Threshold | Replacement ≤ Original | Film capacitors are valued for their low losses. Polypropylene has extremely low tan δ (0.0001–0.0005), while polyester is higher (0.005–0.015). Lower is better. High dissipation factor causes self-heating under AC conditions, which is critical in power electronics, resonant circuits, and EMI filters. This parameter is often the reason polypropylene is specified over polyester. |
| ESR (Equivalent Series Resistance) | Threshold | Replacement ≤ Original | Lower ESR reduces losses and heat generation. Film/foil construction has inherently lower ESR than metallized. ESR is particularly important in snubber circuits, resonant converters, and high-frequency filtering where I²R losses can be significant. |
| dV/dt Rating (Pulse) | Threshold | Replacement ≥ Original | The maximum rate of voltage change the capacitor can withstand without damage. Critical in snubber circuits, commutation circuits, and pulse discharge applications. Specified in V/µs. Film/foil caps handle higher dV/dt than metallized. Exceeding the dV/dt rating can cause localized overheating and dielectric failure. A higher rating is always safer. |
| Peak Pulse Current (Ipeak) | Threshold | Replacement ≥ Original | The maximum instantaneous current the capacitor can handle during a pulse event. Directly related to construction type: film/foil caps handle much higher peak currents than metallized due to thicker electrodes with lower resistance. Critical in snubber, welding, strobe/flash, and pulsed power applications. |
| Ripple Current Rating | Threshold | Replacement ≥ Original | The maximum continuous AC/ripple current at the rated frequency and temperature. Exceeding this causes self-heating and accelerated aging. Particularly important in power factor correction (PFC), motor-run, and AC filtering applications where the capacitor sees continuous AC current. A higher rating provides more thermal margin. |
| Operating Temp Range | Threshold | Replacement range ⊇ Original range | The replacement must cover at least the full operating range of the original. Standard polyester film caps are rated −40°C to +85°C or +105°C; polypropylene typically −55°C to +85°C or +105°C; PPS can reach +150°C. Film capacitors derate significantly at higher temperatures—voltage and ripple current ratings are reduced. A narrower range cannot substitute for a wider one. |
| Temperature Coefficient / Capacitance Stability | Threshold | ¦Replacement¦ ≤ ¦Original¦ | Polypropylene has a very stable, predictable negative temperature coefficient (−200 to −250 ppm/°C). Polyester has a less predictable, larger positive coefficient (+400 to +600 ppm/°C). PPS is near-zero (±30 ppm/°C). For resonant circuits and precision timing, capacitance drift with temperature directly affects frequency stability. Lower magnitude is better. |
| Height (Seated Max) | Fit | Replacement ≤ Original | Film capacitors vary widely in height depending on voltage, capacitance, and construction. Box-type radial caps can be quite tall. Critical for enclosure clearance, especially in compact power supplies and motor drives where film caps are mounted near enclosure walls or fans. |
| Length × Width (Body) | Fit | Replacement ≤ Original | For through-hole box-type film caps, the body dimensions determine board space. A larger body may encroach on adjacent components. For SMD film caps, the footprint must match exactly. Note: switching to a higher-performance dielectric (e.g., PET → PP) at the same capacitance and voltage often results in a larger body—verify physical fit before confirming the substitution. |
| Safety Rating (X/Y Class) | Identity (Flag) | If Original = Class X/Y, Replacement = Same Class or Higher | Film capacitors used in EMI/RFI suppression across or between mains lines carry mandatory safety ratings. X1 (4kV surge), X2 (2.5kV surge): line-to-line. Y1 (8kV surge), Y2 (5kV surge): line-to-ground. The replacement must carry the same or higher safety class. An X2 cap cannot replace an X1 cap. A non-safety-rated cap can never replace a safety-rated one. This is a regulatory compliance requirement (UL, IEC, EN). |
| Flammability Rating (UL 94) | Identity (Flag) | If Original = V-0, Replacement = V-0 | Many safety-rated and power film capacitors require UL 94 V-0 flammability rating on the casing material (self-extinguishing). This is a regulatory requirement, not a performance parameter. If the original carries a V-0 rating, the replacement must also. This is non-negotiable in mains-connected applications. |
| AEC-Q200 Qualification | Identity (Flag) | If Original = Yes, Replacement = Yes | Required for automotive applications. Film capacitors in automotive environments (engine bay, ADAS, EV powertrains) face extreme thermal cycling and vibration. AEC-Q200 qualified parts undergo rigorous testing. A non-qualified part cannot replace a qualified one. |
| Moisture Sensitivity Level (MSL) | Threshold | Replacement ≤ Original | Primarily relevant for SMD film capacitors. MSL 1 (unlimited floor life) is best. Most through-hole film caps are not MSL-sensitive, but surface-mount types may require baking before reflow if MSL > 1. |
| Packaging (Tape & Reel, Bulk, etc.) | Operational | Production Match | For SMD film caps, Tape & Reel is required for automated pick-and-place. For through-hole, Ammo Pack (taped leads) is needed for auto-insertion. Bulk or Box packaging requires manual placement. Ensure compatibility with production line feeders. |

## Logic Type Definitions

Identity: The replacement value must exactly match the original. No deviation allowed.

Identity (Upgrade Only): Must match or be a strictly superior variant. For film dielectrics: PP > PPS > PEN > PET.

Identity (Flag): A boolean or class-based attribute. If the original requires it, the replacement must also have it (or meet the same class level). If the original does not require it, either value is acceptable.

Threshold: A numeric attribute where the replacement must meet or exceed (or be at/below) a boundary defined by the original part’s spec.

Fit: A physical/dimensional constraint. The replacement must physically fit in the same space.

Operational: A non-electrical parameter relevant to manufacturing or supply chain compatibility.

## Film Capacitor-Specific Notes

Dielectric Hierarchy — PP > PPS > PEN > PET: Polypropylene (PP) has the lowest dielectric losses (tan δ ≈ 0.0002), best frequency response, and highest insulation resistance, but the lowest volumetric efficiency and a max temperature around 105°C. Polyester (PET) offers 3–4× higher capacitance density in the same volume, but with 50–100× higher losses. PEN and PPS sit between these extremes, with PPS offering the best thermal performance (up to 150°C). Choose the dielectric based on the dominant requirement: PP for precision/low-loss, PET for density/cost, PPS for high temperature.

Self-Healing vs. Film/Foil: Metallized (self-healing) film capacitors use a thin vacuum-deposited metal layer (typically aluminum or zinc) as the electrode. When a dielectric defect causes a breakdown, the arc energy vaporizes the metal around the fault in microseconds, isolating the defect. The capacitor continues to function with a negligible loss of capacitance. Film/foil capacitors use a discrete metal foil as the electrode. They cannot self-heal, but the thicker electrode gives them much lower ESR and far superior pulse current handling. The choice between metallized and film/foil is application-driven and is not a simple “upgrade.”

X/Y Safety Ratings Are Regulatory Requirements: Film capacitors used in EMI/RFI suppression filters on mains power lines must carry X-class (line-to-line) or Y-class (line-to-ground) safety certifications. These are not optional performance specs—they are legally required by UL, IEC 60384-14, and EN standards. Using a non-safety-rated or lower-class capacitor in a safety-rated position can void product certifications and create a fire or shock hazard. X1 > X2 > X3 (line-to-line); Y1 > Y2 > Y3 > Y4 (line-to-ground). The replacement must be rated at the same class or higher.

AC vs. DC Voltage Ratings: Film capacitors often specify both DC and AC voltage ratings separately. The AC rating accounts for continuous dielectric stress, internal heating from AC losses, and corona onset voltage. It is not simply the DC rating divided by √2. For motor-run, PFC, and AC filter applications, always match the explicitly stated AC voltage rating. Using only the DC rating in an AC application can lead to overheating and premature failure.

Snubber and Pulse Applications: In snubber circuits (used to protect switching devices like IGBTs and thyristors), the capacitor experiences extreme dV/dt and high peak currents. The dV/dt rating, peak current capability, and construction type (film/foil preferred) are the dominant selection criteria, often more important than capacitance or voltage rating. Some manufacturers produce dedicated “snubber” or “pulse” rated film caps with specific dV/dt and peak current guarantees—do not substitute with a general-purpose film cap without verifying these specs.