# Ceramic Capacitors – MLCC (Surface Mount)

DigiKey Family ID: 12

Hard Logic Filters for Replacement Part Validation

A few things that are specific to MLCCs:

- Dielectric/Temperature Characteristic — includes the full upgrade hierarchy (C0G > X7R > X5R > X6S > Y5V > Z5U) with reasoning about Class I vs. Class II behavior

- DC Bias Derating — flagged as "Application Review" rather than a simple threshold, because two identically-specced parts from different manufacturers can derate completely differently under DC bias. This can't be automated from datasheet values alone.

- Flexible Termination — a critical flag for flex-rigid PCBs and large packages prone to flex cracking

- ESR and ESL — both included as threshold filters, with a note that ESR can be application-specific in tuned filter circuits

- Piezoelectric Noise — called out in the notes section since "singing capacitor" issues can disqualify Class II substitutions in audio/analog designs

| Attribute | Logic Type | Matching Rule | Engineering Reasoning |
| --- | --- | --- | --- |
| Capacitance | Identity | Exact Match | A 100nF capacitor must be replaced by a 100nF capacitor. Ensure values are normalized before comparison (e.g., 0.1µF = 100nF = 100,000pF). |
| Package / Case | Identity | Exact Match to Original | The replacement must match the original footprint exactly (e.g., if the original is 0805, the replacement must also be 0805). MLCC pad geometries differ across sizes and are not interchangeable without a board redesign. |
| Voltage Rating | Threshold | Replacement ≥ Original | The replacement must handle at least the same voltage. A 50V cap can replace a 25V one, but not vice versa. Caution: higher voltage-rated MLCCs of the same capacitance/package may exhibit different DC bias derating characteristics. Verify effective capacitance at operating voltage if this is critical. |
| Dielectric / Temperature Characteristic | Identity (Upgrade Only) | Replacement = or ↑ Original | Dielectrics have a strict hierarchy. Class I (C0G/NP0) is the most stable: near-zero DC bias derating, lowest loss, tightest TCCs. Class II (X7R, X5R) offers higher capacitance density but with voltage/temp derating. Upgrade path: C0G > X7R > X5R > X6S > Y5V > Z5U. You can replace X5R with X7R or C0G (upgrade), but never replace C0G with X7R (downgrade). |
| Tolerance | Threshold | Replacement ≤ Original | A tighter tolerance is always acceptable. A ±5% cap can replace a ±10% one, but not vice versa. Note: for MLCCs, common tolerances are ±20% (M), ±10% (K), ±5% (J), ±2% (G), ±1% (F). |
| Operating Temp Range (Min/Max) | Threshold | Replacement range ⊇ Original range | The replacement must cover at least the full operating range of the original. X7R covers −55°C to +125°C; X5R covers −55°C to +85°C. A part rated for a narrower range cannot substitute for one rated wider. Note: the dielectric code encodes this (the first and last characters), but always verify the actual datasheet range. |
| Height (Seated Max) | Fit | Replacement ≤ Original | Critical for tight enclosures, stacked PCBs, and low-profile designs. MLCCs in the same package code can vary in height depending on capacitance and voltage rating. A taller part may not fit. Always compare max seated height values. |
| ESR (Equivalent Series Resistance) | Threshold | Replacement ≤ Original | Lower ESR is generally better for decoupling and filtering. A 10mΩ cap can replace a 50mΩ one in most applications. Exception: in some resonant or EMI filter designs, specific ESR is required. Flag this attribute for review if the circuit is a tuned filter. |
| ESL (Equivalent Series Inductance) | Threshold | Replacement ≤ Original | Lower ESL improves high-frequency decoupling performance. Particularly important for power integrity on high-speed digital boards. Reverse-geometry packages (e.g., 0306 vs. 0603) have significantly lower ESL and are a valid upgrade, provided the footprint is compatible. |
| Flexible Termination | Identity (Flag) | If Original = Yes, Replacement = Yes | Flexible (or “soft”) terminations absorb board flex stress and prevent flex cracking, a common MLCC failure mode. Required on flex-rigid PCBs, large packages (≥0805), and boards subject to mechanical stress. A standard-termination cap cannot replace a flex-term cap in these applications. |
| Moisture Sensitivity Level (MSL) | Threshold | Replacement ≤ Original | Affects manufacturing handling. MSL 1 (unlimited floor life) is best; higher MSL numbers require baking before reflow soldering. A lower MSL number is less restrictive and always acceptable as a replacement. |
| AEC-Q200 Qualification | Identity (Flag) | If Original = Yes, Replacement = Yes | Required for automotive and high-reliability applications. AEC-Q200 certified parts undergo rigorous life testing for moisture, thermal shock, and vibration. A non-qualified part cannot replace a qualified one. If the original does not require it, a qualified replacement is acceptable. |
| DC Bias Derating | Application Review | Verify effective capacitance at Vdc | Class II MLCCs (X7R, X5R) lose significant capacitance under DC bias. A 10µF X5R 0805 at 10V may only deliver 3µF effective. Different manufacturers’ parts with identical specs can derate differently. This cannot be filtered by datasheet value alone—consult the manufacturer’s DC bias curves or use simulation tools (e.g., Murata SimSurfing, TDK SEAT). |
| Packaging (Tape & Reel, etc.) | Operational | Production Match | Not an electrical parameter, but critical for manufacturing. Tape & Reel is required for automated pick-and-place. Cut Tape or Bulk may not be compatible with production lines. Ensure reel width and pitch match feeder specs. |

## Logic Type Definitions

Identity: The replacement value must exactly match the original. No deviation allowed.

Identity (Upgrade Only): Must match or be a strictly superior variant. For dielectrics: C0G > X7R > X5R > X6S > Y5V > Z5U.

Identity (Flag): A boolean attribute. If the original requires it, the replacement must also have it. If the original does not, either value is acceptable.

Threshold: A numeric attribute where the replacement must meet or exceed (or be at/below) a boundary defined by the original part’s spec.

Fit: A physical/dimensional constraint. The replacement must physically fit in the same space.

Application Review: Cannot be fully automated by spec comparison alone. Requires consulting manufacturer curves or simulation tools for proper validation.

Operational: A non-electrical parameter relevant to manufacturing or supply chain compatibility.

## MLCC-Specific Notes

Dielectric Code Decoding: EIA codes encode temperature range and capacitance change. The first character is the low temp (X = −55°C, Y = −30°C), the second is the high temp (5 = +85°C, 7 = +125°C, 8 = +150°C), and the third is the max capacitance change over that range (R = ±15%, S = ±22%, V = +22%/−82%, U = +22%/−56%).

DC Bias Derating: This is the single most common pitfall in MLCC substitution. Two parts with identical datasheet capacitance, voltage, and package can deliver dramatically different effective capacitance under DC bias. Always check manufacturer DC bias curves when substituting Class II dielectrics in voltage-critical applications.

Piezoelectric Noise (Singing Capacitors): Class II MLCCs exhibit a piezoelectric effect that can cause audible noise in audio circuits or sensitive analog designs. If the original design specifies a low-noise or C0G capacitor for this reason, do not substitute with a Class II dielectric.