# Tantalum Capacitors

DigiKey Family ID: 59

Hard Logic Filters for Replacement Part Validation

| Attribute | Logic Type | Matching Rule | Engineering Reasoning |
| --- | --- | --- | --- |
| Capacitance | Identity | Exact Match | A 10µF capacitor must be replaced by a 10µF capacitor. Normalize values before comparison. Tantalum capacitors are available in standard E-series values, typically from 0.1µF to 1000µF. |
| Voltage Rating | Threshold | Replacement ≥ Original | The replacement must handle at least the same voltage. A 16V cap can replace a 10V one. Critical: tantalum capacitors are highly susceptible to voltage-induced failure (ignition). Industry best practice is to derate to 50% of rated voltage in application. A higher voltage rating provides additional safety margin and is always preferred. |
| Package / Case (EIA Size) | Identity | Exact Match to Original | Tantalum SMD packages use EIA case codes: A (3216), B (3528), C (6032), D (7343), E (7360). Each has a unique footprint. The replacement must match exactly—different case codes have different pad geometries and are not interchangeable without board rework. |
| Capacitor Type | Identity (Upgrade Only) | Replacement = or ↑ Original | Tantalum capacitors come in several sub-types: MnO₂ (standard) and polymer. Polymer tantalums have lower ESR, higher ripple current capability, and a benign (non-ignition) failure mode. Upgrade path: MnO₂ → Polymer is acceptable. Downgrading from Polymer → MnO₂ may introduce ignition-mode failure risk in applications that depend on benign failure behavior. |
| Tolerance | Threshold | Replacement ≤ Original | A tighter tolerance is always acceptable. Common tantalum tolerances: ±20% (M), ±10% (K), ±5% (J). A ±10% cap can replace a ±20% one, but not vice versa. |
| ESR (Equivalent Series Resistance) | Threshold | Replacement ≤ Original | ESR is critical for tantalums, especially in power supply decoupling and output filtering. MnO₂ types typically have ESR in the range of 0.5–5Ω, while polymer types are much lower (10–100mΩ). Lower ESR reduces voltage ripple and heat generation. Always compare ESR at the same frequency (typically 100kHz). |
| Ripple Current Rating | Threshold | Replacement ≥ Original | The maximum RMS ripple current the cap can sustain. Exceeding this causes internal heating, which is particularly dangerous in MnO₂ tantalums due to thermal runaway risk. Polymer tantalums tolerate ripple better and fail benignly. A higher ripple current rating is always safer. |
| Leakage Current | Threshold | Replacement ≤ Original | Tantalum capacitors have higher leakage than ceramics but lower than aluminum electrolytics. Typically specified as I = 0.01CV or 0.5µA (whichever is greater) after 5 minutes at rated voltage. Lower leakage is better. Critical in battery-powered and low-power sleep-mode applications. |
| Dissipation Factor (tan δ) | Threshold | Replacement ≤ Original | A measure of dielectric loss, typically specified at 120Hz and 25°C. Lower is better. MnO₂ tantalums typically range from 0.04 to 0.10; polymer tantalums are generally lower. A higher dissipation factor means more energy lost as heat under AC ripple conditions. |
| Operating Temp Range | Threshold | Replacement range ⊇ Original range | The replacement must cover at least the full operating range of the original. Standard tantalums are rated −55°C to +125°C; some high-reliability or polymer types extend to +150°C or +175°C. A narrower range cannot substitute for a wider one. Note that capacitance and ESR both degrade at temperature extremes. |
| Height (Seated Max) | Fit | Replacement ≤ Original | SMD tantalums within the same EIA case code can vary in height depending on capacitance and voltage rating. A taller part may not fit under enclosure lids, heat sinks, or in stacked PCB assemblies. Always compare the maximum seated height, not just the case code. |
| Surge Current / Inrush Protection | Application Review | Verify circuit protection | Tantalum capacitors are notoriously sensitive to surge (inrush) current at power-on. MnO₂ types can fail catastrophically (short and ignite) if subjected to high inrush current. The original design may include a series resistance (≥1Ω/V) or soft-start circuitry to limit inrush. If replacing with a lower-ESR part (especially polymer), verify that the total circuit impedance still limits inrush to safe levels. |
| Failure Mode | Identity (Flag) | If Original = Benign, Replacement = Benign | MnO₂ tantalums fail in short-circuit (ignition) mode: the manganese dioxide cathode provides oxygen, enabling self-sustaining combustion. Polymer tantalums fail in a benign (open-circuit) mode because the polymer cathode does not supply oxygen. If the original design specifies polymer for safety-critical benign failure behavior, do not substitute with MnO₂. |
| DC Bias Derating | Application Review | Verify effective capacitance at Vdc | Tantalum capacitors exhibit less DC bias derating than Class II MLCCs, but it is still present, especially at higher voltages. Polymer tantalums generally derate less than MnO₂ types. When substituting in voltage-critical applications, consult the manufacturer’s DC bias curves to verify effective capacitance at operating voltage. |
| AEC-Q200 Qualification | Identity (Flag) | If Original = Yes, Replacement = Yes | Required for automotive applications. Tantalums in automotive must withstand extended thermal cycling, vibration, and humidity. Given their inherent failure mode risks, automotive-qualified tantalums undergo additional screening. A non-qualified part cannot replace a qualified one. |
| COTS / MIL-PRF-55365 Rating | Identity (Flag) | If Original = MIL, Replacement = MIL | Military and aerospace applications may require MIL-PRF-55365 qualified tantalums with additional screening levels (e.g., T, S, R, P, M failure rate levels). A commercial (COTS) part cannot replace a MIL-spec part. If the original specifies a particular failure rate level, the replacement must meet or exceed it. |
| Moisture Sensitivity Level (MSL) | Threshold | Replacement ≤ Original | Affects assembly handling for SMD tantalums. MSL 1 (unlimited floor life) is best. Higher MSL numbers require baking before reflow to prevent moisture-induced damage (delamination, cracking). A lower MSL number is always acceptable as a replacement. |
| Packaging (Tape & Reel, etc.) | Operational | Production Match | Tape & Reel is required for automated SMD pick-and-place. Cut Tape or Bulk may not be compatible with production feeders. For through-hole (dipped) tantalums, verify lead form and spacing compatibility with the assembly process. |

## Logic Type Definitions

Identity: The replacement value must exactly match the original. No deviation allowed.

Identity (Upgrade Only): Must match or be a strictly superior variant. For tantalums: MnO₂ (standard) → Polymer is an upgrade.

Identity (Flag): A boolean attribute. If the original requires it, the replacement must also have it. If the original does not, either value is acceptable.

Threshold: A numeric attribute where the replacement must meet or exceed (or be at/below) a boundary defined by the original part’s spec.

Fit: A physical/dimensional constraint. The replacement must physically fit in the same space.

Application Review: Cannot be fully automated by spec comparison alone. Requires circuit-level analysis or manufacturer data to validate.

Operational: A non-electrical parameter relevant to manufacturing or supply chain compatibility.

## Tantalum-Specific Notes

Voltage Derating — The 50% Rule: Industry best practice (and many corporate standards) requires tantalum capacitors to be derated to 50% of their rated voltage in application. A 10V-rated tantalum should only see ≤5V in circuit. When selecting replacements, a higher voltage rating provides a larger safety margin. Some manufacturers recommend 60–70% derating for MnO₂ types and allow 80% for polymer types due to their benign failure mode.

Ignition vs. Benign Failure: MnO₂ (manganese dioxide) cathode tantalums can fail catastrophically: a dielectric breakdown creates a short circuit, and the MnO₂ supplies oxygen to sustain a localized fire. Polymer cathode tantalums fail open-circuit because the conductive polymer thermally degrades at the fault site, self-isolating the defect. This fundamental difference in failure mode is the single most important distinction when substituting between tantalum sub-types.

Inrush Current Protection: Tantalums are uniquely vulnerable to damage from inrush (surge) current at power-on, particularly in low-impedance power rails connected directly to a battery or power bus. The original circuit should include series resistance (≥1Ω per volt of rating is a common guideline) or soft-start circuitry. When substituting a lower-ESR part (e.g., replacing MnO₂ with polymer), the total source impedance drops, and inrush current increases—verify that protection is still adequate.

Tantalum vs. MLCC / Aluminum Trade-offs: Tantalums occupy a middle ground: more stable than aluminum electrolytics (no electrolyte dry-out), higher volumetric efficiency than MLCCs at high capacitance, but with unique failure mode risks. Modern high-capacitance MLCCs and polymer aluminum caps are increasingly replacing tantalums in new designs. When evaluating cross-family substitutions (outside the scope of this filter), consider that swapping families changes the entire parameter profile and requires full design review.